Water quality survey of streams across Wales: runoff form forested and felled catchments draining a range of soil types

This data provides the results of a survey of the water quality of small streams draining forested and felled catchments across Wales. The water quality measurements are extensive, including analysis of major, minor, trace and ultra-trace elements together with nutrient and standard water quality measures such as pH and Gran alkalinity. Opportunistic sampling was undertaken with the aid for Forest Enterprise staff to sample sites at periods of both dry and very wet weather in order to assess the water quality under baseflow and stormflow conditions, respectively, to assess groundwater and soil endmember chemistries. The work was undertaken as part of a joint NERC, Environment Agency and Forestry Commission funded study to examine the impacts of conifer harvesting and replanting on upland water quality (Neal et al., 1998). Small catchment sites (2 to 5 ha) were chosen single tree and soil type at each location. Across the sites, the number of samplings varied between 1 and 10 depending upon feasibility of sampling. The monitoring period was from the 7th September 1995 up to the 18th November 1997.

The data presented is “raw” and has not been edited to include less than values etc. This is because the data is directed to the research community. It allows the user to observe a more detailed rather than grainy picture of water quality differences and undertake statistical analysis without the problems that arise when less than values are included. Contained within the metadata is a file that gives all the analytical methods used, including information on detection levels etc. This allows the user to adjust the data to standard protocols for presenting an edited dataset. In addition, metadata files are provided to describe site locations (grid references) and associated information (soil types, tree species and felling dates etc.) as well as field sampling and other protocols prior to chemical analysis.

The scope and range of the Welsh survey work together with the findings are provided by Neal et al., 1998 and we ask that that citation be given to this paper when using and publishing material from this dataset.

Reference

Neal, C., Reynolds, B., Wilkinson, R.J., Hill, T., Neal, M., Hill, S. and Harrow, M.L. 1998. The impacts of conifer harvesting on runoff water quality: a regional survey for Wales. Hydrology and Earth System Sciences, 2 (2-3), 323-344.

Note that this paper is open access and can be obtained for the Hydrology and Earth System Sciences web page (http://www.hydrol-earth-syst-sci.net/title_and_author_search.html?author=neal&x=0&y=0).